Supplementary Documents

# 1. Model framework and state variables

The current model is a transient one-dimensional sediment reactive transport model. It solves coupled transport and reactions for solid and dissolved species on a depth-resolved grid using a finite-volume method-of-lines framework. The resulting ODE system is integrated with MATLAB ode15s under constant forcing for spin-up or under user-defined transient forcing for future applications.

The model represents a 1-D sediment column extending from the sediment-water interface to a prescribed lower boundary depth, L. The vertical coordinate z is positive downward. The sediment column is discretized into n finite-volume grid cells with grid spacing Δz. The coupled model state contains four solid-phase variables and eight dissolved variables.

Y=Ys,YdT  1

where Ys and Yd are the solid-phase and dissolved-phase state vectors, respectively. The state variables are listed in Table 1.

For n depth nodes and 12 state variables, the state vector has length 12n. The transient model is integrated as

dYdt=Ft,Y,  2

where F contains finite-volume transport, boundary exchange, and local reaction source/sink terms.

Table 1. Model state variables.

# 2. Sediment physical structure and transport equations

Depth-dependent physical properties are prescribed from site-level parameters. Porosity, solid mixing, and porewater exchange are represented as exponentially decreasing functions of depth:

ϕz=ϕb+ϕ0-ϕbexp-zLϕ,Dbz=Db,b+Db,0-Db,bexp-zLb,αz=αb+α0-αbexp-zLα.  3

Here, ϕ0 and ϕb are surface and deep porosities, Db,0 and Db,b are surface and deep solid mixing coefficients, and α0 and αb are surface and deep porewater exchange coefficients. The parameters Lϕ, Lb, and Lα are the corresponding attenuation length scales.

The solid burial velocity is adjusted for porosity,

wsz=wb1-ϕb1-ϕz,  4

where wb is the deep solid burial velocity. The organic matter reactivity profile is calculated from sediment age,

azi=a0+j=1iΔzwszj, ksedz=sk10-0.95log10az-0.81,  5

where a0 is the initial surface sediment age and sk is a site-level organic matter reactivity multiplier.

## 2.1 Solid-phase transport

For a solid-phase species Cs, the dry solid mass per bulk sediment volume is

Az=ρ1-ϕz,  6

where ρ is sediment solid density. The conservation equation is

∂AzCs∂t=-∂Fs∂z+AzRs,  7

where Fs is the solid-phase transport flux and Rs is the local solid-phase reaction rate. The solid flux combines bioturbation-like mixing and burial,

Fs=-AzDbz∂Cs∂z+AzwszCs.  8

The upper boundary is imposed as a depositional flux Fs,0, and the lower boundary allows burial outflow with zero diffusive-gradient mixing.

## 2.2 Dissolved-phase transport and finite-volume form

For a dissolved species C, storage is porosity-weighted:

∂ϕzC∂t=-∂F∂z+R+ϕzαzCtop-C.  9

Here, F is the dissolved transport flux, R is the local bulk-volume reaction source/sink term, αz is the exchange coefficient, and Ctop is the overlying-water concentration used for the upper boundary and exchange term. The dissolved flux is

F=-ϕzD∂C∂z+ϕzwfzC,  10

where D is the species-specific effective diffusion coefficient and wfz is porewater advection velocity. In the current baseline configuration, wf=0. The upper boundary is implemented through the finite-volume top face using Ctop, and the lower boundary assumes zero diffusive gradient with possible advective outflow:

C0=Ctop, ∂C∂zz=L=0.  11

For grid cell i, the finite-volume transport tendency is

dSidttrans=-Fi+1/2-Fi-1/2Δz,  12

where Si=AiCi for solid species and Si=ϕiCi for dissolved species. The formulation is conservative because fluxes are exchanged through shared cell faces.

# 3. Organic matter degradation and primary redox cascade

Total organic matter input is prescribed directly or estimated from net primary production and burial efficiency:

FOM,total=BE⋅NPP  13

Here, NPP is net primary production, BE is burial efficiency. Organic matter input is partitioned into labile and refractory pools,

FOM,lab=flab⋅FOM,total, FOM,ref=1-flab⋅FOM,total.  14

The labile fraction flab controls the fraction of organic matter input that is available for redox-active degradation. Temperature dependence is represented using a Q10 correction,

Tf=Q10T-Tref/10,  15

where T is temperature and Tref is the reference temperature. The bulk-volume organic carbon mineralization rate is

RC=Tf⋅ksedz⋅OMlab⋅ρ1-ϕz12×109.  16

The factor 12 converts carbon mass to moles under the implemented model convention, and 109 converts mol cm⁻³ to μmol L⁻¹. The labile pool supplies most redox-active carbon, whereas the refractory pool represents preserved organic matter inventory. In the current baseline configuration, refractory degradation is disabled by setting kref=0.

Aerobic respiration is calculated as

RO2,resp=RC⋅O2O2+KO2,  17

where KO2 is the oxygen half-saturation constant. The remaining carbon is sequentially allocated to Fe reduction, sulfate reduction, and methanogenesis.

Local FeOOH availability is represented by

fFe=FeOOHFeOOH+KFe, RFeRedpot=4⋅RC,1⋅fFe.  18

Here, KFe is the FeOOH availability constant, and RC,1 is the residual carbon after aerobic respiration. To separate local FeOOH inventory from sustained external Fe supply, potential Fe reduction is capped using

IFeRedpot=10-30LRFeRedpot dz, IFeext=FFeOx, RFeRed=min1,IFeextIFeRedpot⋅RFeRedpot.  19

Sulfate reduction and methanogenesis are then represented as

RSRR=0.5RC,2⋅SO4SO4+KSO4, RMeth=0.5⋅RC,3,  20

where KSO4 is the sulfate half-saturation constant, RC,2 is residual carbon after Fe reduction, and RC,3 is residual carbon after sulfate reduction. Sulfate-dependent reactions are shut off when SO4≤SO4,cut, where SO4,cut is a numerical depletion threshold.

# 4. Secondary Fe-S-CH4 reactions and carbonate chemistry

Total sulfide is converted to a free sulfide fraction using pH:

HSfree=HStot1+106-pH/KHS,  21

where KHS controls the pH-dependent free sulfide fraction. Fe(II) oxidation, FeS formation, sulfide oxidation, AOM, and aerobic methane oxidation are represented by

RFeOx=kFeOx⋅FeIIO2,RFeS=kFeS⋅FeII⋅HSfree,RHSOx=kHSOx⋅HSfree⋅O2,RAOM=kAOM⋅CH4⋅SO4SO4+KCH4,SO4,RCH4Ox=kCH4Ox⋅CH4⋅O2O2+KCH4,O2.  22

The associated secondary oxygen sink is

RO2,sec=0.25RFeOx+2RHSOx+2RCH4Ox.  23

Methane ebullition is represented as a dissolved methane loss term above an effective threshold:

RBubble=kBubble⋅maxCH4-CH4,thr,0.  24

Carbonate speciation is calculated from DIC and alkalinity. The model uses

DIC=H2CO3+HCO3+CO3, ALK=HCO3+2CO3+BOH4+OH-H.  25

Calcite saturation is represented as

σcarb=Ω-1=γCa⋅Ca2+⋅γCO3⋅CO32-Ksp-1,  26

where γCa and γCO3 are activity coefficients and Ksp is the calcite solubility product. Net carbonate reaction is represented by Rcarb, with positive values indicating precipitation and negative values indicating dissolution.

# 5. Net reaction source-sink ledgers

The reaction terms passed to the transport solver combine the process-level rates above. For the solid phases,

ROMlab=-Tf⋅ksed⋅OMlab,ROMref=-Tf⋅kref⋅ksed⋅OMref,RFeOOH=-RFeRed+RFeOx10-3ρ1-ϕ,RCaCO3=Rcarbs.  27

For the dissolved phases,

RO2=-RO2,resp-RO2,sec,RFeII=RFeRed-RFeOx-RFeS,RSO4=-RSRR-RAOM+RHSOx,RHS=RSRR+RAOM-RFeS-RHSOx,RCH4=RMeth-RAOM-RCH4Ox-RBubble,RDIC=RO2,resp+RFeRed4+2RSRR+RMeth+RCH4Ox+RAOM-Rcarb,RALK=0.5RFeRed+2RSRR+2RAOM-2RFeS-RHSOx-2Rcarb,RCa=-Rcarb.  28

These ledgers define how process rates enter each model state variable. They are written as local reaction terms and are combined with finite-volume transport and boundary exchange in the right-hand-side calculation.

# 6. Boundary forcing, initialization, and numerical implementation

Upper boundary concentrations are specified for dissolved species as

Ctop=O2,top,Fetop,SO4,top,HStop,CH4,top,DICtop,ALKtop,Catop.  29

Solid depositional fluxes are specified for organic matter, FeOOH, and CaCO3. The FeOOH inventory flux is scaled as

FFeOOH,0=fFe,inv36.5FFeOx,  30

whereas the integrated Fe supply cap uses 36.5FFeOx without applying the inventory scaling factor. This distinction separates the current reactive FeOOH inventory from the external reactive Fe input flux.

The transient model starts from simple nonnegative initial profiles and is integrated under constant forcing until it approaches a quasi-steady profile. The method-of-lines formulation is implemented by packing all state variables and all depth-grid values into Y, evaluating dY/dt, and integrating the resulting stiff ODE system with a nonnegative constraint. The spin-up duration is a numerical equilibration period rather than a literal environmental scenario duration.

# 7. Baseline parameters and model development notes

The values below are current baseline or initial test values from the present model configuration. They should be treated as documentation of the current model setup rather than final calibrated parameters.

Table 2. Baseline physical and forcing parameters.

Table 3. Baseline transport and reaction parameters.

# 8. Planned river-system parameterization

A next model-development step is to link the one-dimensional sediment-column parameters to measurable riverbed and hydrologic properties. Relevant river-system descriptors include median grain size D50, porosity, sediment organic matter content, sediment accumulation or exchange rate, hydraulic exchange with surface water or groundwater, and oxygen supply.

In future river applications, porosity parameters ϕ0,ϕb,Lϕ, exchange coefficients αz, and sediment burial or exchange velocity ws may be parameterized as functions of sediment texture, permeability, water depth, bed shear stress, channel slope, and groundwater-surface water exchange. A conceptual parameterization is

α0=fαD50,Kperm,ustar,H,S,qgw,  31

where Kperm is sediment permeability, ustar is shear velocity, H is water depth, S is channel slope, and qgw is groundwater exchange flux. Similarly, sediment burial or exchange could be expressed as

ws=fwQs,τb,τc,D50,  32

where Qs is sediment supply, τb is bed shear stress, and τc is critical shear stress for erosion or resuspension. These parameterizations are placeholders for future regional river applications rather than finalized model equations.